Massachusetts Institute of Technology

Department of Electrical Engineering and Computer Science

6.111 Introductory Digital Systems Laboratory (Spring 2007)

Final Project Check Off Sheet

Project Title: Real-time Raytracing

Student Names: Sam Gross, Adam Lerer

TA Name: Javier Castro

TA Signature/Date:

System Level

| Display the rendered image in real-time at 640x480 @ 75 Hz and 1024x768 @ 60Hz using a double buffer |  |
| --- | --- |
| Shader performs object shading using ambient, diffuse, and specular shading models |  |
| Master control unit calculates eye rays for each pixel, passes them to raytracing units, and reconstructs frames from the raytracing units. |  |
| The raytracer will achieve a frame rate of 20fps, with partial credit for 5fps or 1 fps |  |

Intersector and Ray-tracing Unit (Adam)

| State Transition Diagrams, Block Diagrams, and code for Intersector and Raytracing unit. |  |
| --- | --- |
| Intersector detects the closest intersection of a ray with a set of spheres and planes |  |
| Raytracing unit receives an eye ray and calculates the color of the eye ray, including reflections. |  |

Fractional Divider and Vector Normalizer (Sam)

| State Transition Diagrams, Block Diagrams, and code for Fractional Divider and Vector Normalizer. |  |
| --- | --- |
| Fast fractional divider can divide two 18-bit numbers in less than 7 clock cycles at 85MHz. |  |
| Vector normalizer uses Newton’s method to do a fast inverse square root for normalization. |  |