Global fitting algorithm:

This algorithm has been developed to fit fluorescence lifetime images. It allows to fit each pixel in the image with a double exponential decay. The two time constants are the same for the whole image and only the contribution of each of them to these decay changes.

See Pelet et al. Biophys. J. 2004, 87, 2807-2817 for more details on the algorithm.

This .zip file contains four .m files and one .mat file:

fit_IC_global.m

gen_array.m

Data_segmentation.m

dblexpconv_fgh.m

test_data.mat

To test the algorithm: open test_data.mat which contains three matlab variables (Data, IR, coef). Data is a 2D matrix containing the lifetime image data. The first line is the time axis while the other lines represent the evolution of the fluorescence as a function of time in each pixel of the image. IR (1D array) is the impulse response function which is given as function of the same time axis as the data. Coef is a 1D array containing the coefficients used to generate the image. The two first coefficients represent the lifetime (1/). The rest of the array represent the initial intensity (coef2i+1) of pixel i and the ratio between the two lifetime (coef2i+2) at the same pixel.

Fit_IC_global is the global fitting program. To test it, type :

[coef_fit, Data_fit, iter_evol] = fit_IC_global(Data, IR, 4);

It should converge quickly. On our computer (Intel Xeo 2.4 GHz, 1GB of RAM) it reaches convergence in 6 seconds.

If you want to change the pre-fitting optimization to use a data segmentation change the 4 to 1 (intensity based segmentation) or 2 (lifetime based segmentation).

Data_segmentation and dblexpconv_fgh are two sub-functions from the main function. You can use Data_segmentation to prepare segmentation and and save them as mat file to prevent any user prompt during the algorithm.

Dblexpconv_fgh calculates the 2 value and the gradient and Hessian for each step of the optimization.

You can use gen_array to generate some new data sets with more pixels and different average photon counts levels.